ANNEX A

Proposed Amendments on Section 4.3.4 of WESM Manual on Constraint Violation Coefficients and Pricing Re-Runs

| Provision | Proposed Amendment |
| --- | --- |
| 4.3.4 The following table provides the constraint violation coefficients, which is reflective of the order of relaxing soft constraints established in Section 4.3.1 of this Market Manual, and the corresponding action by the System Operator: Order; Constraint Violation Coefficient Name; CVC; SO Action. 1; Tertiary Reserve Requirement Constraint; 100,000; None. 2; Primary Reserve Requirement Constraint; 200,000; None. 3; Nodal VoLL or Nodal Energy Balance Constraint; 800,000; Re-dispatch generation and/or drop load as necessary.. 4; System Energy Balance Constraint; 1,300,000; For over-generation, identify generating units to be shut down to eliminate excess capacity.. For under-generation, identify must-run units that can be dispatched or drop load as necessary.. 5; Self-Scheduled Generation Constraint; 1,400,000; The projected output or schedule of loading level of the relevant generating unit(s) shall be curtailed.. 6; Thermal Contingency Constraint – Transformer; 1,500,000; Re-dispatch generation and/or drop load as necessary.. 7; Thermal Contingency Constraint – Line; 1,500,000. 8; Thermal Contingency Constraint – Branch Group; 2,000,000. 9; Secondary Reserve Requirement Constraint; 3,500,000; Re-dispatch generation and/or drop load as necessary.. 10; Thermal Base Case Constraint – Transformer; 4,000,000; Re-dispatch generation and/or drop load as necessary.. 11; Thermal Base Case Constraint – Line; 4,000,000. 12; Thermal Base Case Constraint – Branch Group; 4,500,000. | 4.3.4 The following table provides the constraint violation coefficients, which is reflective of the order of relaxing soft constraints established in Section 4.3.1 of this Market Manual, and the corresponding action by the System Operator: Order; Constraint Violation Coefficient Name; CVC; SO Action. 1; Tertiary Reserve Requirement Constraint; 100,000; None. 2; Primary Reserve Requirement Constraint; 200,000; None. 3; Nodal VoLL or Nodal Energy Balance Constraint; 800,000; Re-dispatch generation and/or drop load as necessary.. 4; System Energy Balance Constraint; 1,300,000; For over-generation, identify generating units to be shut down to eliminate excess capacity.. For under-generation, identify must-run units that can be dispatched or drop load as necessary.. 5; Self-Scheduled Generation Constraint – Non-Scheduled Generation; 1,400,000; The projected output or schedule of loading level of the relevant non-scheduled generating unit(s) shall be curtailed.. 6; Self-Scheduled Generation Constraint – Priority Dispatch Generation; 1,500,000; The projected output or schedule of loading level of the relevant priority dispatch generating unit(s) shall be curtailed.. 7; Self-Scheduled Generation Constraint – Must Dispatch Generation; 1,600,000; The projected output or schedule of loading level of the relevant must dispatch generating unit(s) shall be curtailed.. 6 8; Thermal Contingency Constraint – Transformer; 1,500,000 2,500,000; Re-dispatch generation and/or drop load as necessary.. 7 9; Thermal Contingency Constraint – Line; 1,500,000 2,500,000. 8 10; Thermal Contingency Constraint – Branch Group; 2,000,000 3,000,000. 9 11; Secondary Reserve Requirement Constraint; 3,500,000 5,600,000; Re-dispatch generation and/or drop load as necessary.. 10 12; Thermal Base Case Constraint – Transformer; 4,000,000 6,000,000; Re-dispatch generation and/or drop load as necessary.. 11 13; Thermal Base Case Constraint – Line; 4,000,000 6,000,000. 12 14; Thermal Base Case Constraint – Branch Group; 4,500,000 6,500,000. |
| 5.3.1 The corresponding constraint relaxation formulas for the constraint violation coefficients during pricing re-runs shall be as provided in Table 2 below: Order; Constraint Violation Coefficient Name; CVC; Violation Variable Value; Delta; Constraint Relaxation during Pricing Re-Run; Re-run Price. 1; Tertiary Reserve Requirement Constraint; 100,000; x; 0.1; x + delta; EDP AND RP. 2; Primary Reserve Requirement Constraint; 200,000; x; 0.1; x + delta; EDP AND RP. 3; Nodal Energy Balance Constraint; 800,000; x; 0.1; x + delta; EDP AND RP. 4; System Energy Balance Constraint; 1,300,000; x; 0; delta; Excess Price for over-generation. Shortage Price for under-generation. 5; Self-Scheduled Generation Constraint; 1,400,000; x; 0.1; x + delta; EDP AND RP. 6; Thermal Contingency Constraint – Transformer; 1,500,000; x; 0.1; x + delta; EDP AND RP. 7; Thermal Contingency Constraint – Line; 1,500,000; x; 0.1; x + delta; EDP AND RP. 8; Thermal Contingency Constraint – Branch Group; 2,000,000; x; 0.1; x + delta; EDP AND RP. 9; Secondary Reserve Requirement Constraint; 3,500,000; x; 0.1; x + delta; EDP AND RP. 10; Thermal Base Case Constraint – Transformer; 4,000,000; x; 0.1; x + delta; EDP AND RP. 11; Thermal Base Case Constraint – Line; 4,000,000; x; 0.1; x + delta; EDP AND RP. 12; Thermal Base Case Constraint – Branch Group; 4,500,000; x; 0.1; x + delta; EDP AND RP. | 5.3.1 The corresponding constraint relaxation formulas for the constraint violation coefficients during pricing re-runs shall be as provided in Table 2 below: Order; Constraint Violation Coefficient Name; CVC; Violation Variable Value; Delta; Constraint Relaxation during Pricing Re-Run; Re-run Price. 1; Tertiary Reserve Requirement Constraint; 100,000; x; 0.1; x + delta; EDP AND RP. 2; Primary Reserve Requirement Constraint; 200,000; x; 0.1; x + delta; EDP AND RP. 3; Nodal Energy Balance Constraint; 800,000; x; 0.1; x + delta; EDP AND RP. 4; System Energy Balance Constraint; 1,300,000; x; 0; delta; Excess Price for over-generation. Shortage Price for under-generation. 5; Self-Scheduled Generation Constraint – Non-Scheduled Generation; 1,400,000; x; 0.1; x + delta; EDP AND RP. 6; Self-Scheduled Generation Constraint – Non-Priority Dispatch Generation; 1,500,000; x; 0.1; x + delta; EDP AND RP. 7; Self-Scheduled Generation Constraint – Must Dispatch Generation; 1,600,000; x; 0.1; x + delta; EDP AND RP. 6 8; Thermal Contingency Constraint – Transformer; 1,500,000 2,500,000; x; 0.1; x + delta; EDP AND RP. 7 9; Thermal Contingency Constraint – Line; 1,500,000 2,500,000; x; 0.1; x + delta; EDP AND RP. 8 10; Thermal Contingency Constraint – Branch Group; 2,000,000 3,000,000; x; 0.1; x + delta; EDP AND RP. 9 11; Secondary Reserve Requirement Constraint; 3,500,000 5,600,000; x; 0.1; x + delta; EDP AND RP. 10 12; Thermal Base Case Constraint – Transformer; 4,000,000 6,000,000; x; 0.1; x + delta; EDP AND RP. 11 13; Thermal Base Case Constraint – Line; 4,000,000 6,000,000; x; 0.1; x + delta; EDP AND RP. 12 14; Thermal Base Case Constraint – Branch Group; 4,500,000 6,500,000; x; 0.1; x + delta; EDP AND RP. |