ORCP-WR-WM-20-04

REQUEST FOR AMENDMENTS OR CHANGES TO THE WESM MANUALS

Proposals made only under this prescribed form shall be accepted and considered as submitted:

This request for amendments to the WESM Rules can be submitted to:

PEM Board

Attention: PEM Committee Secretariat

Philippine Electricity Market Corporation

18/F Robinsons Equitable Tower

ADB Avenue, Ortigas Center

Pasig City, 1605 Philippines

Email address: rcc@wesm.ph

Fax Number: (+632) 395-2704

- Proponent’s Information

| Name | Richard J. Nethercott |
| --- | --- |
| Designation | President and CEO |
| Company | Independent Electricity Market Operator of the Philippines, Inc. |
| Company Address | 19/F Robinsons Equitable Tower ADB Ave. Ortigas Center |
|  | Pasig City |
| Telephone No. | 02 318 9376 |
| Fax. No. | 02 636 0802 |
| Email Address | richard.nethercott@iemop.ph |

- WESM Manual Amendments Information

Title of WESM Manual being commented: Management of Net Settlement Surplus Issue 3.0 Nature of Request (please indicate with x) Addition Alteration Deletion Clarification Clerical Correction

- Proposed Amendment

| Title | Section | Provision | Proposed Amendment | Rationale | Comments | Proposed Wording based on Comments | Proponent’s Response | RCC Agreements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| References | 2.2 | This document should be read together with the – XXX e) Energy Regulatory Commission Resolution No. 6, Series of 2009, A Resolution Adopting the Rules for the Distribution of Net Settlement Surplus. | This document should be read together with the – XXX e) Energy Regulatory Commission Resolution No. 67, Series of 20092019, A Resolution Adopting Amendments to the Rules for the Distribution of Net Settlement Surplus (NSS). | To refer to the latest ERC issuance amending the NSS Rules |  |  |  |  |
| Responsibilities | 3.1.1 | In accordance with WESM Rules Clause 3.13.12.3, the Market Operator shall: XXX | In accordance with WESM Rules Clause 3.13.1216.3, the Market Operator shall: XXX | To reflect the appropriate clause number of the WESM Rules providing the responsibilities of the Market Operator on the NSS |  |  |  |  |
| Ownership of Net Settlement Surplus | 4.2 | OWNERSHIP OF NET SETTLEMENT SURPLUS | OWNERSHIP OF NET SETTLEMENT SURPLUS RE-DISTRIBUTION OF NET SETTLEMENT SURPLUS/DEFICIT | To adopt the wording of Article IV of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| Ownership of Net Settlement Surplus | 4.2 | 4.2.1 WESM Participants that paid for the loss and congestion charge shall receive a share in the net settlement surplus. Pursuant to ERC Resolution No. 01 Series of 2018, the net settlement surplus (NSS) shall be allocated to Direct WESM Members as follows: a) Distribution Utilities (DUs); b) Retail Energy Suppliers (RES), including Local RES and Supplier of Last Resort (SoLR) and other electricity suppliers; c) Generation Companies that acted as end-users or made withdrawal from the grid; d) Independent Power Producer (IPP) Administrators who will be assigned Contracts for the Supply of Electric Energy (CSEE) and assume the default wholesale supply functions for the assigned TSC Customers; and e) Other parties which have acted as end-users and paid line loss and congestion charges. 4.2.2 Any NSS allocated to the DUs shall be subject to immediate re-distribution to the corresponding End-users at the retail level. 4.2.3 Any NSS allocated to the RES shall be subject to immediate re-distribution to the corresponding End-users in the retail electricity market. 4.2.4 Any NSS allocated to a Direct WESM Member on behalf of its Indirect WESM Member shall be subject to immediate re-distribution to the corresponding Indirect WESM Member. 4.2.5 When prices in the WESM are administered i.e., upon the occurrence of Market Intervention, Market Suspension, or implementation of Secondary Price Cap, there shall be no NSS allocation and re-distribution for the affected trading intervals. | 4.2.1 Pursuant to Article IV Section 1 of ERC Resolution No. 7 Series of 2019, the Net Settlement Surplus (NSS) or Net Settlement Deficit (NSD) shall be billed to Direct WESM Members including, but not limited to, the following entities: - Distribution Utilities (DUs); - Retail Electricity Suppliers (RES), including Local RES and the Supplier of Last Resort (SoLR), and other electricity suppliers; - Generation Companies; - IPP Administrators; and - Other parties registered as Direct WESM Members 4.2.2 The total of the hourly NSS and/or NSD allocation for every billing period shall be reflected immediately as a deduction to or an adjustment to the total settlement amount of the Direct WESM Member, whether for its own WESM transactions or on behalf of its Indirect WESM Member. This shall be correspondingly re-distributed to the end-users at the retail level as part of the monthly generation rate pursuant to their corresponding power supply agreements approved by the Energy Regulatory Commission. 4.2.3 NSS/NSD allocation corresponding to adjustments for transactions prior to the switching of an Indirect WESM Member to another Direct Member shall be billed to the current Direct WESM Member. 4.2.4 Any NSS/NSD allocation billed to the RES shall be subject to re-distribution to its Contestable Customers pursuant to their Retail Supply Contract. WESM Participants that paid for the loss and congestion charge shall receive a share in the net settlement surplus. Pursuant to ERC Resolution No. 01 Series of 2018, the net settlement surplus (NSS) shall be allocated to Direct WESM Members as follows: a) Distribution Utilities (DUs); b) Retail Energy Suppliers (RES), including Local RES and Supplier of Last Resort (SoLR) and other electricity suppliers; c) Generation Companies that acted as end-users or made withdrawal from the grid; d) Independent Power Producer (IPP) Administrators who will be assigned Contracts for the Supply of Electric Energy (CSEE) and assume the default wholesale supply functions for the assigned TSC Customers; and e) Other parties which have acted as end-users and paid line loss and congestion charges. 4.2.2 Any NSS allocated to the DUs shall be subject to immediate re-distribution to the corresponding End-users at the retail level. 4.2.3 Any NSS allocated to the RES shall be subject to immediate re-distribution to the corresponding End-users in the retail electricity market. 4.2.4 Any NSS allocated to a Direct WESM Member on behalf of its Indirect WESM Member shall be subject to immediate re-distribution to the corresponding Indirect WESM Member. 4.2.5 When prices in the WESM are administered i.e., upon the occurrence of Market Intervention, Market Suspension, or implementation of Secondary Price Cap, there shall be no NSS allocation and re-distribution for the affected trading intervals. | For harmonization with Article IV of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| Methodology and Procedure | 5.1 | CALCULATION OF NET SETTLEMENT SURPLUS | CALCULATION OF NET SETTLEMENT SURPLUS AND NET SETTLEMENT DEFICIT | For harmonization with ERC Resolution No. 7 Series of 2019, which also includes separate provisions for the calculation of Net Settlement Deficit |  |  |  |  |
| Methodology and Procedure | 5.1.1 | The adoption of locational marginal pricing for the WESM under different market trading nodes accounts for congestion and losses in the transmission system. As a result, the price differences occur between generator nodes and customer nodes due to the losses and congestion. The surplus resulting from the aggregate WESM settlement transactions in the Net Settlement Surplus. This may also result in a settlement deficit. | The adoption of locational marginal pricing for the WESM under different market trading nodes accounts for congestion and losses in the transmission system. As a result, the price differences occur between generator nodes and customer nodes due to the losses and congestion. The surplus resulting from the aggregate WESM settlement transactions in the Net Settlement Surplus. This may also result in a settlement deficit. The aggregate WESM settlement amount may result in: 1) Net Settlement Surplus which is also known as NSS; or 2) Net Settlement Deficit which is also known as NSD. | For harmonization with Article II Section 1 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| Methodology and Procedure | 5.1.1.1 (new) | (new) | The Net Settlement Surplus or Net Settlement Deficit amount shall be calculated based on the total trading amount for every trading interval. | For harmonization with Article II of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| Methodology and Procedure | 5.1.1.2 (new) | (new) | The Net Settlement Surplus occurs when the total collectibles exceed the total payables. | For harmonization with Article II of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| Methodology and Procedure | 5.1.1.3 (new) | (new) | The Net Settlement Deficit occurs when the total payables exceed the total collectibles. | For harmonization with Article II of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| Methodology and Procedure | 5.2 | Pursuant to Article II Section 2 of the ERC Resolution No. 01 Series of 2018, the net settlement surplus amount shall be calculated based on the total trading amount for every trading interval as follows: NSSk = Collectiblesk – Payablesk Where: NSSk = Net Settlement Surplus for trading interval k Collectibles = total amount to be collected by the Market Operator from the Trading Participants for energy transactions in the market (including loss and congestion charges for both spot and bilateral quantities) for all nodes at trading interval k Payables = total amount to be paid by the Market Operator to the Trading Participants for energy transactions in the market (including loss and congestion charges) for all nodes at trading interval k k = hourly trading interval | Pursuant to Article II Section 2 of the ERC Resolution No. 01 Series of 2018 Article III Section 1 of ERC Resolution No. 07 Series of 2019, the net settlement surplus amount NSSTotal/NSDTotal shall be determined by getting the difference of the total collectibles (TACollectibles) and the total payables (TAPayables): calculated based on the total trading amount for every trading interval as follows NSSk = Collectiblesk –Payablesk NSSTotal,i/NSDTotal,i = TACollectibles,i - TAPayables,i Where: NSSk = Net Settlement Surplus for trading interval k Collectibles = total amount to be collected by the Market Operator from the Trading Participants for energy transactions in the market (including loss and congestion charges for both spot and bilateral quantities) for all nodes at trading interval k Payables = total amount to be paid by the Market Operator to the Trading Participants for energy transactions in the market (including loss and congestion charges) for all nodes at trading interval k k = hourly trading interval NSSTotal/NSDTotal = Total Net Settlement Surplus/Deficit, in PhP, for trading interval i TACollectibles = Total trading amount, in PhP, to be collected by the Market Operator from the Trading Participants for trading interval i TAPayables = Total trading amount, in PhP, to be paid by the Market Operator to the Trading Participants for trading interval i i = trading interval | For harmonization with Article III Section 1 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DERIVATION OF NET SETTLEMENT SURPLUS AND NET SETTLEMENT DEFICIT | 5.2 | NET SETTLEMENT DEFICIT | NET SETTLEMENT DEFICIT DERIVATION OF NET SETTLEMENT SURPLUS AND NET SETTLEMENT DEFICIT | For harmonization with Article III of ERC Resolution No. 7 Series of 2019. The original Section 5.2 is suggested to be replaced by the proposed revisions and the proposed new provisions of section 5.2 of this manual. |  |  |  |  |
| DERIVATION OF NET SETTLEMENT SURPLUS AND NET SETTLEMENT DEFICIT | 5.2.1 | The net settlement surplus may be retained by the Market Operator to fund deficit settlement, pursuant to WESM Rules Clause 3.13.16.2(a). Since the issuance of the settlement statements are on monthly basis, the formula for calculation of the net settlement surplus presented in Section 5.1 of this document automatically takes the deficit into account. Thus, the resulting calculation presented in the statements already netted out the deficit. | Pursuant to Article III Section 2 of ERC Resolution No. 07 Series of 2019, allocation of the NSS/NSD shall be based on the cause of the surplus (e.g., surplus transmission loss payment, surplus congestion payment, among others). As such, the NSS/NSD calculation is disaggregated into NSS/NSD due to losses (NSSLoss) and the NSS/NSD due to congestion (NSSCongestion), which are allocated separately. The following formula illustrates the components of the disaggregated NSS/NSD: NSSTotal,D,i/NSDTotal,D,i = NSS/NSDLoss,i + NSS/NSDCongestion,i NSSTotal,D,i/NSDTotal,D,i = Sum of disaggregated Net Settlement Surplus/Deficit due to transmission loss and congestion, in PhP, for trading interval i NSSLoss,i/NSDLoss,i = Net Settlement Surplus/Deficit due to transmission loss payments, in PhP, for trading interval i NSSCongestion,i/NSDCongestion,i = Net Settlement Surplus/Deficit due to congestion payments, in PhP, for trading interval i i = trading interval i The net settlement surplus may be retained by the Market Operator to fund deficit settlement, pursuant to WESM Rules Clause 3.13.16.2(a). Since the issuance of the settlement statements are on monthly basis, the formula for calculation of the net settlement surplus presented in Section 5.1 of this document automatically takes the deficit into account. Thus, the resulting calculation presented in the statements already netted out the deficit. | For harmonization with Article III of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DERIVATION OF NET SETTLEMENT SURPLUS AND NET SETTLEMENT DEFICIT | 5.2.2 | It is possible that a net settlement deficit rather than a surplus will occur. That is, for certain trading intervals the sum of the trading amounts at the generator nodes (i.e., payments) is greater than the trading amounts at the load or Customer nodes (i.e., collections). This deficit may occur because of differences between the ex-ante and ex-post energy trading amounts, which are due, in turn, to un-realized schedules and resulting price differences during real time (i.e., ex-post). These occurrences are beyond the control of the Market Operator and are brought about by offer and bid structures submitted to the market by the Trading Participants. | In order to determine NSSLoss/NSDLoss and NSSCongestion/NSDCongestion , the trading amounts shall also be disaggregated among its components. Based on the components of the locational marginal prices (LMP), the trading amounts collected and paid by the Market Operator for each trading participant shall be disaggregated as follows: TAPayable,P,i/TACollectible,P,i = TAEnergy,P,i + TALoss,P,i + TACongestion,P,i Where: TAPayable,P,i/TACollectible,P,i = Trading amount of Trading Participant P, in PhP, for trading interval i TAEnergy,P,i = Trading amount for energy of Trading Participant P, in PhP, for trading interval i TALoss,P,i = Trading amount for transmission loss of Trading Participant P, in PhP, for trading interval i TACongestion,P,i = Trading amount for congestion of Trading Participant P, in PhP, for trading interval i It is possible that a net settlement deficit rather than a surplus will occur. That is, for certain trading intervals the sum of the trading amounts at the generator nodes (i.e., payments) is greater than the trading amounts at the load or Customer nodes (i.e., collections). This deficit may occur because of differences between the ex-ante and ex-post energy trading amounts, which are due, in turn, to un-realized schedules and resulting price differences during real time (i.e., ex-post). These occurrences are beyond the control of the Market Operator and are brought about by offer and bid structures submitted to the market by the Trading Participants. | For harmonization with Article III of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DERIVATION OF NET SETTLEMENT SURPLUS AND NET SETTLEMENT DEFICIT | 5.2.2.1 (new) | (new) | Trading amount for energy (TAEnergy) shall be computed as follows: TAEnergy,p,i=EAQp,i-c∈CBCQp,c,i* λRTD,p,i+MQp,i-EAQp,i*λRTX,p,i +c∈CBCQp,c,i*(λRTD,p,i-λRTD,s,i) Where: λRTD/RTX,p,i = Applicable ex-ante (RTD) or ex-post (RTX) system marginal price (SMP) of resource p for trading interval i λRTD,s,i = Ex-ante (RTD) system marginal price (SMP) of selling counterparty resource s (on the bilateral contract between resource p and counterparty c) for trading interval i EAQp,i = Ex-ante quantity, in MWh, of resource p for trading interval i BCQp,c,i= Bilateral contract quantity, in MWh, of the bilateral contract between resource p and counterparty c for trading interval i MQp,i= Metered quantity, in MWh, of between resource p trading interval i c = Counterparty of resource p C = Set of all counterparties of resource p p = Any resource of Trading Participant P i = trading interval | For harmonization with Article III of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DERIVATION OF NET SETTLEMENT SURPLUS AND NET SETTLEMENT DEFICIT | 5.2.2.2 (new) | (new) | Trading amount for transmission loss (TALoss) shall be computed as follows: TALoss,p,i=EAQp,i-c∈CBCQp,c,i* MTLPRTD,p,i+MQp,i-EAQp,i*MTLPRTX,p,i +c∈CBCQp,c,i*(MTLPRTD,p,i-MTLPRTD,s,i) Where: MTLPRTD/RTX,p,i = Applicable ex-ante (RTD) or ex-post (RTX) marginal transmission loss price (MTLP) of resource p for trading interval i MTLPRTD,s,i = Ex-ante (RTD) marginal transmission loss price (MTLP) of selling counterparty resource s (on the bilateral contract between resource p and counterparty c) for trading interval i EAQp,i = Ex-ante quantity, in MWh, of resource p for trading interval i BCQp,c,i= Bilateral contract quantity, in MWh, of the bilateral contract between resource p and counterparty c for trading interval i MQp,i= Metered quantity, in MWh, of between resource p trading interval i c = Counterparty of resource p C = Set of all counterparties of resource p p = Any resource of Trading Participant P i = trading interval |  |  |  |  |  |
| DERIVATION OF NET SETTLEMENT SURPLUS AND NET SETTLEMENT DEFICIT | 5.2.2.3 (new) | (new) | Trading amount for congestion (TACongestion) shall be computed as follows: TACongestion,p,i=EAQp,i-c∈CBCQp,c,i* MCPRTD,p,i+MQp,i-EAQp,i*MCPRTX,p,i +c∈CBCQp,c,i*(MCPRTD,p,i-MCPRTD,s,i) Where: MCPRTD/RTX,p,i = Applicable ex-ante (RTD) or ex-post (RTX) marginal congestion price (MCP) of resource p for trading interval i MCPRTD,s,i = Ex-ante (RTD) marginal congestion price (MCP) of selling counterparty resource s (on the bilateral contract between resource p and counterparty c) for trading interval i EAQp,i = Ex-ante quantity, in MWh, of resource p for trading interval i BCQp,c,i= Bilateral contract quantity of the bilateral contract between resource p and counterparty c for trading interval i MQp,i= Metered quantity, in MWh, of between resource p trading interval i c = Counterparty of resource p C = Set of all counterparties of resource p p = Any resource of Trading Participant P i = trading interval | For harmonization with Article III of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DERIVATION OF NET SETTLEMENT SURPLUS AND NET SETTLEMENT DEFICIT | 5.2.3 (new) | (new) | Net Settlement Surplus/Deficit due to line loss payment (NSSLoss/NSDLoss) shall be determined as the difference of the collectibles and the payables for energy and transmission loss as shown be the equation below: NSSLoss,i/NSDLoss,i = cp∈CP(TAEnergy,cp,i+TALoss,cp,i)- pp∈PP(TAEnergy,pp,i+TALoss,pp,i) Where: NSSLoss,i/NSDLoss,i = Net Settlement Surplus/Deficit due to line loss payments, in PhP, for trading interval i TAEnergy = Total trading amount of a trading participant for energy, in PhP, for trading interval i TALoss = Total trading amount of a Trading Participant for transmission loss, in PhP, for trading interval i CP = Set of Trading Participants with collectible trading amounts cp = Trading Participant with collectible trading amount PP = Set of Trading Participants with payable trading amounts pp = Trading Participant with payable trading amount i = trading interval | For harmonization with Article III of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DERIVATION OF NET SETTLEMENT SURPLUS AND NET SETTLEMENT DEFICIT | 5.2.4 (new) | (new) | Net Settlement Surplus/Deficit due to congestion (NSSCongestion/NSDCongestion) shall be calculated as the difference of the collectibles and the payables for congestion as shown below: NSSCongestion,i/NSDCongestion,i = cp∈CPTACongestion,cp,i- pp∈PPTACongestion,pp,i Where: NSSCongestion,i/NSDCongestion,i = Net Settlement Surplus/Deficit due to congestion payments, in PhP, for trading interval i TACongestion = Total trading amount of a trading participant for congestion, in PhP, for trading interval i CP = Set of Trading Participants with collectible trading amounts cp = Trading Participant with collectible trading amount PP = Set of Trading Participants with payable trading amounts pp = Trading Participant with payable trading amount i = trading interval | For harmonization with Article III of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DERIVATION OF NET SETTLEMENT SURPLUS AND NET SETTLEMENT DEFICIT | 5.2.5 (new) | (new) | During trading intervals when there is no computed transmission loss and congestion, the NSS/NSD shall be computed as follows: NSSNoTLCP,i/NSDTotal,i = TAcollectibles,i - TApayables,i Where: NSSNoTLCP,i/NSDNoTLCP,i = Total Net Settlement Surplus/Deficit when there is no calculated transmission loss and congestion payments, in PhP, for trading interval i TAcollectibles,i = Total trading amount, in PhP, to be collected by the Market Operator from the Trading Participants for trading interval i TApayables,i = Total trading amount, in PhP, to be paid by the Market Operator to the Trading Participants for trading interval i i = trading interval | For harmonization with Article III of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3 | FLOW BACK TO THE PARTICIPANTS | FLOW BACK TO THE PARTICIPANTS DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | To adopt the calculations provided under Article V of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.1 | The net settlement surplus after funding the deficit settlement will be returned to the WESM Participants. The allocation shall be on a pro-rata basis depending on each recipient’s contribution to the total NSS. The monthly amount to be allocated to each recipient shall be equal to the sum of the recipient’s NSS allocation amounts computed for all trading intervals in a billing month. A recipient’s NSS allocation amount per interval is equal to the total net settlement surplus amount per trading interval multiplied by the ratio of the recipient’s line loss and congestion charges payments for the trading interval to the total line loss and congestion charges payments for the trading interval of all recipients. This rule shall apply regardless of the allocation method, whether outright deduction or otherwise. | The Market Operator shall allocate the NSS to the Trading Participants that paid for the surplus loss and congestion charges. The allocation shall be determined based on each recipient’s contribution to the total NSSLoss and NSSCongestion. The net settlement surplus after funding the deficit settlement will be returned to the WESM Participants. The allocation shall be on a pro-rata basis depending on each recipient’s contribution to the total NSS. The monthly amount to be allocated to each recipient shall be equal to the sum of the recipient’s NSS allocation amounts computed for all trading intervals in a billing month. A recipient’s NSS allocation amount per interval is equal to the total net settlement surplus amount per trading interval multiplied by the ratio of the recipient’s line loss and congestion charges payments for the trading interval to the total line loss and congestion charges payments for the trading interval of all recipients. This rule shall apply regardless of the allocation method, whether outright deduction or otherwise. | For harmonization with Article V Section 1 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.2 | This is represented by the following formula: RJk = NSSk x (Σ𝐋𝐋𝐂𝐂𝒋,𝒌 / Σ𝐋𝐋𝐂𝐂𝒌) Where: RJ,k = rebate amount or NSS allocation for Participant J for trading interval k NSSk = Net Settlement Surplus for the trading interval k Σ𝐋𝐋𝐂𝐂𝒋,𝒌 = sum of the line loss and congestion charges payments of Participant J’s resources for the trading interval k Σ𝐋𝐋𝐂𝐂𝒌 = sum of line loss and congestion charges payments of all eligible resources for the trading interval k J = any WESM Participant paying line loss congestion charges for which a pro-rated amount of NSS will be returned or allocated j = any resource of Participant J paying line loss and congestion charges for which a line loss and congestion charge payment will be computed k = trading interval | The allocation of NSSLoss shall be calculated as follows: NSSLoss,p,i=LLSpot,p,i + LLLR,p,iLLSpot,i + LLLR,i × NSSLoss,i Where: NSSLoss,p,i = NSS allocation due to line loss payments, in PhP, of resource p for trading interval i LLSpot,p,i = Surplus line loss payment for spot transaction, in PhP, of resource p for trading interval i LLLR,p,i = Surplus line loss payment for line rental, in PhP, of resource p for trading interval i NSSLoss,i = Net Settlement Surplus due to line loss payments, in PhP, for trading interval i p = Any resource of Trading Participant P i = trading interval This is represented by the following formula: RJk = NSSk x (Σ𝐋𝐋𝐂𝐂𝒋,𝒌 / Σ𝐋𝐋𝐂𝐂𝒌) Where: RJ,k = rebate amount or NSS allocation for Participant J for trading interval k NSSk = Net Settlement Surplus for the trading interval k Σ𝐋𝐋𝐂𝐂𝒋,𝒌 = sum of the line loss and congestion charges payments of Participant J’s resources for the trading interval k Σ𝐋𝐋𝐂𝐂𝒌 = sum of line loss and congestion charges payments of all eligible resources for the trading interval k J = any WESM Participant paying line loss congestion charges for which a pro-rated amount of NSS will be returned or allocated j = any resource of Participant J paying line loss and congestion charges for which a line loss and congestion charge payment will be computed k = trading interval | For harmonization with Article V Section 1 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.2.1 (new) | (new) | 5.3.2.1 LLSpot and LLLR shall be determined using the following equations: - LLSpot,p,i= MAXEAQp,i,c∈CBCQp,c,i-c∈CBCQp,c,i×MTLPRTD,p,i-GWAMTLPRTD,i+MQp,i-EAQp,i×MTLPRTX,p,i-GWAMTLPRTX,i - LLLR,p,i= c∈C{BCQp,c,i×[MTLPRTD,p,i-MAXGWAMTLPRTD,i, MTLPRTD,s,i]} Where: LLSpot,p,i = Surplus line loss payment for spot transaction, in PhP, of resource p for trading interval i LLLR,p,i = Surplus line loss payment for line rental, in PhP, of resource p for trading interval i EAQp,i = Ex-ante quanity, in MWh, of resource p for trading interval i BCQp,c,i= Bilateral contract quantity, in MWh, of the bilateral contract between resource p and counterparty c for trading interval i MTLPRTD,p,i= Ex-ante marginal transmission loss price, in PhP/MWh, of resource p for trading interval i GWAMTLPRTD,i= Ex-ante generator-weighted average marginal transmission loss price, in PhP/MWh, for trading interval i MQp,i= Metered quantity, in MWh, of resource p for trading interval i MTLPRTX,p,i= Ex-post marginal transmission loss price, in PhP/MWh, of resource p for trading interval i GWAMTLPRTX,i= Ex-post generator-weighted average marginal transmission loss price, in PhP/MWh, for trading interval i MTLPRTD,s,i= Ex-ante marginal transmission loss price, in PhP/MWh, of selling counterpart s (on the bilateral contract of resource p and counterparty c) for trading interval i - GWAMTLPi= g∈GMTLPg,i×Gen. Scheduleg,ig∈GGen. Scheduleg,i Where: GWAMTLPi= Ex-ante (RTD) or ex-post (RTX) generator-weighted average marginal transmission loss price, in PhP/MWh, for trading interval i MTLPg,i= Marginal transmission loss price, in PhP/MWh, of generator resource g for trading interval i Gen. Scheduleg,i= Scheduled injection, in MW, of generator resource g for trading interval i g = Generator resource G= Set of all generator resources i= trading interval | For harmonization with Article V Section 1 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.2.2 (new) | (new) | 5.3.2.2 LLSpot shall only be calculated if the Trading Participant had a withdrawal of energy from the grid. The Trading Participant is deemed to have a withdrawal from the grid if its metered quantity (MQ) or ex-ante quantity (EAQ) is less than zero (0). | For harmonization with Article V Section 1 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.2.3 (new) | (new) | 5.3.2.3 LLLR shall only be calculated if the Trading Participant is the assigned counterparty for the line rental. | For harmonization with Article V Section 1 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.2.4 (new) | (new) | 5.3.2.4 If the calculated LLSpot and/or LLLR does not reflect surplus payment (i.e., LLSpot / LLLR is greater than zero (0)), the corresponding LLSpot / LLLR shall be set to zero (0). | For harmonization with Article V Section 1 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.3 | The value of the total net settlement surplus, ΣNSSk, is derived from the hourly settlement runs. | The allocation of NSSCongestion shall be calculated as follows: NSSCongestion,p,i=CCSpot,p,i + CCLR,p,iCCSpot,i + CCLR,i × NSSCongestion,i Where: NSSCongestion,p,i = NSS allocation due to congestion charge payments, in PhP, of resource p for trading interval i CCSpot,p,i = Surplus congestion charge payment for spot transaction, in PhP, of resource p for trading interval i CCLR,p,i = Congestion charge payment for line rental, in PhP, of resource p for trading interval i NSSCongestion,i = Net Settlement Surplus due to congestion payments, in PhP, for trading interval i p = Any resource of Trading Participant P i = trading interval The value of the total net settlement surplus, ΣNSSk, is derived from the hourly settlement runs. | For harmonization with Article V Section 1 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.3.1 (new) | (new) | CCSpot and CCLR shall be determined using the following equations: - CCSpot,p,i= MAXEAQp,i,c∈CBCQp,c,i-c∈CBCQp,c,i×MCPRTD,p,i-GWAMCPRTD,i+MQp,i-EAQp,i×MCPRTX,p,i-GWAMCPRTX,i - CCLR,p,i= c∈C{BCQp,c,i×[MCPRTD,p,i-MAXGWAMCPRTD,i, MCPRTD,s,i]} Where: CCSpot,p,i = Surplus congestion charge payment for spot transaction, in PhP, of resource p for trading interval i CCLR,p,i = Congestion charge payment for line rental, in PhP, of resource p for trading interval i EAQp,i = Ex-ante quantity, in MWh, of resource p for trading interval i BCQp,c,i= Bilateral contract quantity, in MWh, of the bilateral contract between resource p and counterparty c for trading interval i MCPRTD,p,i= Ex-ante marginal congestion price, in PhP/MWh, of resource p for trading interval i GWAMCPRTD,i= Ex-ante generator-weighted average marginal congestion price, in PhP/MWh, for trading interval i MQp,i= Metered quantity, in MWh, of resource p for trading interval i MCPRTX,p,i= Ex-post marginal congestion price, in PhP/MWh, of resource p for trading interval i GWAMCPRTX,i= Ex-post generator-weighted average marginal congestion price, in PhP/MWh, for trading interval i MCPRTD,s,i= Ex-ante marginal congestion price, in PhP/MWh, of selling counterpart s (on the bilateral contract of resource p and counterparty c) for trading interval i - GWAMCPi= g∈GMCPg,i×Gen. Scheduleg,ig∈GGen. Scheduleg,i Where: GWAMCPi= Ex-ante (RTD) or ex-post (RTX) generator-weighted average marginal congestion price, in PhP/MWh, for trading interval i MCPg,i= Marginal congestion price, in PhP/MWh, of generator resource g for trading interval i Gen. Scheduleg,i= Scheduled injection, in MW, of generator resource g for trading interval i g = Generator resource G= Set of all generator resources i= trading interval | For harmonization with Article V Section 1 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.3.2 (new) | (new) | CCSpot shall only be calculated if the Trading Participant had a withdrawal of energy from the grid. The Trading Participant is deemed to have a withdrawal from the grid if its metered quantity (MQ) or ex-ante quantity (EAQ) is less than zero (0). | For harmonization with Article V Section 1 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.3.3 (new) |  | CCLR shall only be calculated if the Trading Participant is the assigned counterparty for the line rental. | For harmonization with Article V Section 1 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.3.4 (new) | (new) | If the calculated CCSpot and/or CCLR does not reflect surplus payment (i.e. CCSpot / CCLR is greater than zero (0)), the corresponding CCSpot / CCLR shall be set to zero (0). | For harmonization with Article V Section 1 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT SURPLUS ALLOCATION | 5.3.4 | The line loss and congestion charges payments of each Trading Participant shall be calculated as follows: LLCCj,k = LLCPRTDj,k × (EAQj,k – BCQj,c,k) + LLCPRTXj,k × (MQj,k – EAQj,k) + Line Rentalj,k The line loss and congestion price of each trading participants for the ex-ante or ex-post run shall be calculated as follows: LLCPRTD/X,j,k = LMPRTD/X,j,k – MCPLowest,k Where: LLCCj,k = line loss and congestion charges payments of resource j for the trading interval k LLCPRTD/X,j,k = line loss and congestion price of resource j during ex-ante or ex-post run for trading interval k LMPRTD/X,j,k = locational marginal price of resource j during ex-ante or ex-post run for trading interval k MCPlowest,k = lowest marginal clearing price for trading interval k EAQj,k = ex-ante quantity of resource j for trading interval k BCQj,c,k = bilateral contract quantity of the bilateral contract between resource j and counterparty c for trading interval k MQj,k = metered quantity of resource j for trading interval k Line Rentalj,c,k = line rental trading amount associated with the bilateral contract between resource j and counterparty c for trading interval k (this amount may be zero if the resource is not the assigned payer of the line rental trading amount) j = any resource of Participant J paying line loss and congestion charges for which a line loss and congestion charge payment will be computed k = trading interval | In trading intervals when there are no calculated transmission loss and congestion payments the NSS shall be allocated to all trading participants with withdrawal from the grid. The allocation shall be proportionate to the actual withdrawal from the grid of the Trading Participant relative to the total actual withdrawals during the trading interval as shown by the equation below: NSSNoTLCP,p,i=MQWithdrawal,p,iww∈WWMQWithdrawal,ww,i × NSSNoTLCP,i Where: NSSNoTLCP,p,i = NSS allocation when there is no calculated transmission loss and congestion payments, in PhP, of resource p for trading interval i MQWithdrawal,p,i = Actual withdrawal from the grid, in MWh, of resource p for trading interval i NSSNoTLCP,i = Net Settlement Surplus when there is no calculated transmission loss and congestion payments, in PhP, for trading interval i p = Any resource of Trading Participant P ww = Trading Participant with actual withdrawal from the grid WW = Set of all Trading Participants with actual withdrawal from the grid i = trading interval The line loss and congestion charges payments of each Trading Participant shall be calculated as follows: LLCCj,k = LLCPRTDj,k × (EAQj,k – BCQj,c,k) + LLCPRTXj,k × (MQj,k – EAQj,k) + Line Rentalj,k The line loss and congestion price of each trading participants for the ex-ante or ex-post run shall be calculated as follows: LLCPRTD/X,j,k = LMPRTD/X,j,k – MCPLowest,k Where: LLCCj,k = line loss and congestion charges payments of resource j for the trading interval k LLCPRTD/X,j,k = line loss and congestion price of resource j during ex-ante or ex-post run for trading interval k LMPRTD/X,j,k = locational marginal price of resource j during ex-ante or ex-post run for trading interval k MCPlowest,k = lowest marginal clearing price for trading interval k EAQj,k = ex-ante quantity of resource j for trading interval k BCQj,c,k = bilateral contract quantity of the bilateral contract between resource j and counterparty c for trading interval k MQj,k = metered quantity of resource j for trading interval k Line Rentalj,c,k = line rental trading amount associated with the bilateral contract between resource j and counterparty c for trading interval k (this amount may be zero if the resource is not the assigned payer of the line rental trading amount) j = any resource of Participant J paying line loss and congestion charges for which a line loss and congestion charge payment will be computed k = trading interval | For harmonization with Article V Section 2 of ERC Resolution No. 7 Series of 2019. |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4 (new) | (new) | 5.4 DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | To adopt the calculations provided under Article VI of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.1 (new) | (new) | 5.4.1 The Market Operator shall allocate the NSS to 1) Customers who caused the deficit; and /or 2) Customers who received excess payment. The allocation shall be determined based on each recipient’s contribution to the total NSDLoss and NSDCongestion. | For harmonization with Article VI Section 1 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.2 (new) | (new) | 5.4.2 The allocation of NSDLoss shall be calculated as follows: NSDLoss,p,i=LLSpot,p,i + LLLR,p,iLLSpot,i + LLLR,i × NSDLoss,i Where: NSDLoss,p,i = NSD allocation due to line loss payments, in PhP, of resource p for trading interval i LLSpot,p,i = Surplus line loss payment for spot transaction, in PhP, of resource p for trading interval i LLLR,p,i = Surplus line loss payment for line rental, in PhP, of resource p for trading interval i NSDLoss,i = Net Settlement Deficit due to line loss payments, in PhP, for trading interval i p = Any resource of Trading Participant P i = trading interval | For harmonization with Article VI Section 2 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.2.1 (new) | (new) | 5.4.2.1 The computation for LLSpot,p,i and LLLR,p,i shall follow the formulas provided in Section 5.3.2.1 of this Market Manual. | For harmonization with Article VI Section 2 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.2.2 (new) | (new) | 5.4.2.2 LLSpot shall only be calculated if the Trading Participant had a withdrawal of energy from the grid. The Trading Participant is deemed to have a withdrawal from the grid if its metered quantity (MQ) or ex-ante quantity (EAQ) is less than zero (0). | For harmonization with Article VI Section 2 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.2.3 (new) | (new) | 5.4.2.3 LLLR shall only be calculated if the Trading Participant is the assigned counterparty for the line rental. | For harmonization with Article VI Section 2 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.2.4 (new) | (new) | 5.4.2.4 If the calculated LLSpot and/or LLLR did not contribute to the deficit (i.e., LLSpot / LLLR is less than zero (0)), the corresponding LLSpot / LLLR shall be set to zero (0). | For harmonization with Article VI Section 2 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.3 (new) | (new) | 5.4.3 The allocation of NSDCongestion shall be calculated as follows: NSDCongestion,p,i=CCSpot,p,i + CCLR,p,iCCSpot,i + CCLR,i × NSDCongestion,i Where: NSDCongestion,p,i = NSD allocation due to congestion charge payments, in PhP, of resource p for trading interval i CCSpot,p,i = Surplus congestion charge payment for spot transaction, in PhP, of resource p for trading interval i CCLR,p,i = Congestion charge payment for line rental, in PhP, of resource p for trading interval i NSDCongestion,i = Net Settlement Deficit due to congestion payments, in PhP, for trading interval i p = Any resource of Trading Participant P i = trading interval | For harmonization with Article VI Section 2 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.3.1 (new) | (new) | 5.4.3.1 The computation for CCSpot,p,i and CCLR,p,i shall follow the formulas provided in Section 5.3.3.1 of this Market Manual. | For harmonization with Article VI Section 2 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.3.2 (new) | (new) | 5.4.3.2 CCSpot shall only be calculated if the Trading Participant had a withdrawal of energy from the grid. The Trading Participant is deemed to have a withdrawal from the grid if its metered quantity (MQ) or ex-ante quantity (EAQ) is less than zero (0). | For harmonization with Article VI Section 2 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.3.3 (new) | (new) | 5.4.3.3 CCLR shall only be calculated if the Trading Participant is the assigned counterparty for the line rental. | For harmonization with Article VI Section 2 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.3.4 (new) | (new) | 5.4.3.4 If the calculated CCSpot and/or CCLR did not contribute to the deficit (i.e. CCSpot / CCLR is less than zero (0)), the corresponding CCSpot / CCLR shall be set to zero (0). | For harmonization with Article VI Section 2 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| DETERMINATION OF NET SETTLEMENT DEFICIT ALLOCATION | 5.4.4 (new) | (new) | 5.4.4 In trading intervals when there are no calculated transmission loss and congestion payments (e.g. administered pricing, price substitution methodology and secondary price cap, among others) the NSD shall be allocated to all trading participants with actual withdrawal from the grid. The allocation shall be proportionate to the actual withdrawal from the grid of the Trading Participant relative to the total actual withdrawals during the interval as shown by the equation below: NSDNoTLCP,p,i=MQWithdrawal,p,iww∈WWMQWithdrawal,ww,i × NSDNoTLCP,i Where: NSDNoTLCP,p,i = NSD allocation when there is no calculated transmission loss and congestion payments, in PhP, of resource p for trading interval i MQWithdrawal,p,i = Actual withdrawal from the grid, in MWh, of resource p for trading interval i NSDNoTLCP,i = Net Settlement Deficit when there is no calculated transmission loss and congestion payments, in PhP, for trading interval i p = Any resource of Trading Participant P ww = Trading Participant with actual withdrawal from the grid WW = Set of all Trading Participants with actual withdrawal from the grid i = trading interval | For harmonization with Article VI Section 3 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| LIMITATION OF FLOW BACK MECHANISM | 5.4 5.5 | 5.4 LIMITATION OF FLOW BACK MECHANISM | 5.45 LIMITATION OF FLOW BACK MECHANISM | Update numbering of sections |  |  |  |  |
| SCHEDULE OF PAYMENTS TO THE PARTICIPANTS | 5.5 5.6 | 5.5 SCHEDULE OF PAYMENTS TO THE PARTICIPANTS | 5.56 SCHEDULE OF PAYMENTS TO THE PARTICIPANTS | Update numbering of sections |  |  |  |  |
| NET SETTLEMENT SURPLUS RETENTION | 6 | NET SETTLEMENT SURPLUS RETENTION 6.1 XXX 6.2 XXX | NET SETTLEMENT SURPLUS RETENTION 6.1 XXX 6.2 XXX | Provisions for the retention of NSS was removed in previous version of Amended Rules for the Distribution of Net Settlement Surplus (ERC Resolution No. 1 Series of 2018). |  |  |  |  |
| FUND MANAGEMENT AND TAXATION | 7 | FUND MANAGEMENT AND TAXATION 7.1 XXX 7.2 XXX | FUND MANAGEMENT AND TAXATION 7.1 XXX 7.2 XXX | Likewise, provisions for the fund management of retained NSS was removed in previous version of Amended Rules for the Distribution of Net Settlement Surplus (ERC Resolution No. 1 Series of 2018). |  |  |  |  |
| REPORTORIAL REQUIREMENTS | 8.1 | 8.1.1 The Market Operator shall prepare the following: a) Regular monthly summary reports on the amount of NSS or deficit being generated and distributed and the corresponding final metered quantities, Marginal Line Loss and Marginal Congestion Cost. The corresponding value added tax (VAT) and interest amounts, as well as the date when the interests were earned or credited to the market operator, if any. This report shall be made available to all WESM Participants and shall be published in the market information website. A verified copy of the report shall likewise be submitted to the ERC on a monthly basis including contributors to and reasons for the deficit or surplus; and b) Annual report comparing the subject year and the preceding year’s NSS levels and allocations, analysis of the factors and constraints giving rise to any NSS or deficit. This report shall be made available to the WESM Participants and will be submitted to the PEM Board Directors and the ERC. | 8.1.1 The Market Operator shall prepare the following: a) Regular monthly summary reports on the amount of NSS or NSD deficit being generated and distributed to each Trading Participant. and the corresponding final metered quantities, Marginal Line Loss and Marginal Congestion Cost. The corresponding value added tax (VAT) and interest amounts, as well as the date when the interests were earned or credited to the market operator, if any. This report shall be made available to all WESM Participants and shall be published in the market information website. A verified copy of the report shall likewise be submitted to the ERC on a monthly basis including contributors to and reasons for the surplus or deficit or surplus, no later than ninety (90) business days from the last day of the billing period covered by the monthly summary report. The corresponding Value Added Tax (VAT) and interest amounts, as well as the date when the interests were earned or credited to the market operator, if any, shall similarly be included in the subject monthly summary report; and b) The pertinent data covered by the subject monthly summary report shall also be submitted in MS Excel format; and bc) Annual report comparing the subject year and the preceding year’s NSS levels and allocations, analysis of the factors and constraints giving rise to any NSS or deficit. This report shall be made available to the WESM Participants and will be submitted to the PEM Board Directors and the ERC. The annual report on the NSS/NSD calculation and allocation shall be submitted no later than the 30th day of May immediately following the year in review, comparing the subject year and the preceding year’s NSS levels and allocations, and analysis of the factors and constraints giving rise to any NSS or NSD. This report shall also be submitted to the market participants and to the PEM Board. | For harmonization with Article VII Section 1 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| REPORTORIAL REQUIREMENTS | 8.1.2 | The Market Operator shall contract a qualified external auditor who shall conduct an annual audit of the NSS or deficit, the corresponding value added tax (VAT) and interest amounts as well as the procedure used by the Market Operator, and submit to the ERC the audit report immediately upon its completion but no later than three (3) months after the calendar year end. | The Market Operator shall contract a qualified external auditor who shall conduct an annual audit of the NSS or deficit, the corresponding value added tax (VAT) and interest amounts as well as the procedure used by the Market Operator, and submit to the ERC the audit report immediately upon its completion but no later than three (3) months after the calendar year end. The Market Operator shall comply with Clause 5.2.6 of the WESM Rules which requires, among others, the conduct of a spot market audit covering such items as enumerated under Clauses 5.2.6.1 to 5.2.6.4 of the WESM Rules. The audit shall also include the actual calculation of NSS or NSD and its allocation, corresponding VAT and interest amounts, as well as the actual billing and collection. Such audit shall be conducted by a qualified third party auditor, and a quarterly report resulting therefrom shall be submitted to the ERC no later than ninety (90) business days after the billing period in review. | For harmonization with Article VII Sections 2 and 3 of ERC Resolution No. 7 Series of 2019 |  |  |  |  |
| REPORTORIAL REQUIREMENTS | 8.1.3 | DUs and RES, which are eligible recipients of any NSS amount shall also submit monthly report to the ERC of said amount and metered quantities including the corresponding report on NSS re-distributed to customers. For Indirect WESM Members, the responsibility of submitting report shall be borne by the corresponding Direct WESM Members representing the former. | DUs and RES, which are eligible recipients of any NSS amount shall also submit monthly report to the ERC of said amount and metered quantities including the corresponding report on NSS re-distributed to customers. For Indirect WESM Members, the responsibility of submitting report shall be borne by the corresponding Direct WESM Members representing the former. | This requirement is no longer provided in the ERC Resolution No. 7 Series of 2019 |  |  |  |  |

Note: For convenience, please underline and put in bold letters the proposed changes to the WESM Manual.